### Vertical timing information

Mode name          Lines   line    sync      back        active     front     whole frame

Total   width   pulse     porch        time      porch        period

(us)   (us)(lin) (us)(lin)  (us)  (lin)  (us)(lin)  (us)  (lin)

VGA 640x480 60Hz    525    31.78   63   2   953   30   15382  484   285    9   16683  525

### Horizonal timing information

Mode name          Pixel      sync      back  active  front whole line

clock      pulse     porch  time   porch  period

(MHz)   (us)  (pix)  (pix)  (pix)  (pix)  (pix)

VGA 640x480 60Hz   25.175   3.81   96     45    646    13     800

"640 x 350 (EGA on VGA)"    "640 x 400 VGA text"        "VGA industry standard"

Clock frequency 25.175 MHz  Clock frequency 25.175 MHz  Clock frequency 25.175 MHz

Line  frequency 31469 Hz    Line  frequency 31469 Hz    Line  frequency 31469 Hz

Field frequency 70.086 Hz   Field frequency 70.086 Hz   Field frequency 59.94 Hz

One line:                   One line:                   One line:

8 pixels front porch        8 pixels front porch        8 pixels front porch

96 pixels horizontal sync   96 pixels horizontal sync   96 pixels horizontal sync

40 pixels back porch        40 pixels back porch        40 pixels back porch

8 pixels left border        8 pixels left border        8 pixels left border

640 pixels video            640 pixels video            640 pixels video

8 pixels right border       8 pixels right border       8 pixels right border

---                         ---                         ---

800 pixels total per line   800 pixels total per line   800 pixels total per line

One field:                  One field:                  One field:

31 lines front porch         5 lines front porch         2 lines front porch

2 lines vertical sync       2 lines vertical sync       2 lines vertical sync

54 lines back porch         28 lines back porch         25 lines back porch

6 lines top border          7 lines top border          8 lines top border

350 lines video             400 lines video             480 lines video

6 lines bottom border       7 lines bottom border       8 lines bottom border

---                         ---                         ---

449 lines total per field   449 lines total per field   525 lines total per field

Sync polarity: H positive,  Sync polarity: H negative,  Sync polarity: H negative,

V negative                  V positive                  V negative

Scan type: non interlaced.  Scan type: non interlaced.  Scan type: non interlaced.